DRIVER TRAINING UPDATE (8 Hours)

COURSE GOAL:

The course will provide the trainee with the minimum topics of Driver Training/Awareness required in the POST Perishable Skills Training Program including: Basic Driving Principles, Legal and Moral Aspects, Defensive Driving and Maneuvering Course Exercises.

The course consists of a hands-on/practical Driver Training/Awareness for in-service officers.

DRIVER TRAINING/AWARENESS

Minimum Topics/Exercises:

- Behind the wheel exercises to improve driving skills - judgment and decision  making

- Class Exercises/Student Evaluation/Testing

- Defensive driving

- Intersections exercise(s)

- Backing/parking exercise(s)

- Policy, legal and moral issues

- Vehicle Dynamics

COURSE OBJECTIVES:

The trainee will:

1.	Demonstrate knowledge of their Driver Training/Awareness skills and techniques.

2.	Demonstrate a minimum standard of psychomotor skills with every technique and

- exercise, to include:

A.	Judgment

- Decision Making

- Defensive Driving

- Basic Driving Principles

- Policy, Legal and Moral Issues

- Vehicle Dynamics

Minimum standards of performance shall be tested by an instructor observing the trainee during their performance of each technique and exercise.  If the trainee does not meet minimum standards, as established by the presenter, remediation will be provided until the standard is met.

- I.          INTRODUCTION/ORIENTATION

A.	Introduction and Orientation

B.	Course Objectives/Overview/Exercises, Evaluation/Testing

C.	Course Safety Rules at Sonoma County Airport

- II.         BASIC DRIVING PRINCIPLES							   II(g)

A.     Weight Transfer

- 1.	Weight distributed between front and rear wheels

- Engine location has greater part of weight distribution

- Types of weight transfer

  - Lateral: Side to side

  - Longitudinal: Front to rear/Rear to front

- Lateral transfer created when vehicle turned left/right

- Longitudinal transfer created when:

  - Braking - Rear/front

  - Accelerating - Front/rear

  - Decelerating - Rear/front

6.	Cannot be eliminated in a moving vehicle

7.	Minimized by good driving techniques and smooth operation

B.	     Steering Control

1.	     Seating position

- Driver Comfort

- Efficient vehicle control

- Wrist break over top of steering wheel

- Seated 12" from air bag

- Adjust mirrors

2.        Steering method - Two hand shuffle steering

- Hand position at 8 and 4 o’clock positions

- Hands do not leave steering wheel

- Maximizes steering accuracy

- Safer and more effective recovery

- Minimizes weight transfer

- Minimizes air bag deployment injury

3.        Steering method - Backing				        		II(e)

- Body rotated to right

- Right hand placed on right headrest

- Vision directed over right shoulder

- Left hand on steering wheel at twelve o’clock position

- Press left leg against bottom of steering wheel for stability

- Left foot braced on floorboard

C.        Roadway Position

1.        Definition: The position of the vehicle on the roadway that maximizes                         speed with minimum steering and risk of loss of vehicle control while

negotiating a turn.

a.    AKA  Driving line

2.        Driving advantages

a.    Minimize and control weight transfer

b.    Minimize steering input

c.    Smoother vehicle operation

d.    Maximum speed through turns in the safest manner

3.        Driving points in a turn

a.   Entry (Point #1)

b.   Apex (Point #2)

c.   Exit (Point #3)

III.        LEGAL AND MORAL ASPECTS					                      	II(f)

A.	      California Codes

1.        17001 CVC

2.        17004 CVC

3.        17004.7 CVC

4.        21052 CVC

5.        21055 CVC

6.        21056 CVC

7.        21057 CVC

8.        21806 CVC

9.        21807 CVC

10.       22350 CVC

B.		Case Law

C.  	Review of Santa Rosa Police Department Policies

1.	Lexipol 316, Code 3 Emergency Vehicle Operation			Policy 316

2.	Lexipol 314, Vehicle Pursuit Policy					Policy 314

D.     Moral aspects

IV.         	DEFENSIVE DRIVING						                        II(c)

A.	Defensive Drivers

1.         Avoid collisions regardless of right-of-way

2.         React properly to hazards

3.         Maintain a professional attitude

B. 	Dangerous Driver Attitudes

1.	Overconfidence

2.	Self-righteousness

3.	 Impatience

4.	Preoccupation

5.         Judgement

6.	Decision making

C. 	Collision Avoidance

1.      Space cushion

a.	Three second rule

b.	Perception / Reaction time

c.	When stopped, see rear wheels

2.	Intersections						                    II(d)

a.	Clear left, right, then left again

b.	Cover brake on stale green

c.	Don’t turn wheels until ready for turn

d.	Look through turns

3.        Maintain high visual horizon with psychomotor awareness

4.        Consider steering to the rear of a conflict vehicle

5.        Backing

a.	Large percentage of collisions involving LE vehicles

b.	Use proper backing techniques

6.      Lane changes

a.	Signal

7.      Blind spots

D.	Occupant Safety Devices

1.        Safety belts

2.        Supplemental Restraint System (air bags)

E. 	Vehicle Dynamics							  	II(g)

1.        Rear wheel cheat

2.        Front-end swing

3.        Oversteer

4.        Understeer

- V.         MANEUVERING COURSE EXERCISES					           II(a,b)

A.   Core 5 exercises

1.	Offset Lane Maneuver

2.	Steering Course Exercise

3.	T-Driveway Exercise

4.	Turn around maneuver

5.	Parallel parking							II(e)

6.	Garage exercise

B.	Controlled Speed Course

1.	Weight transfer

2.	Braking techniques

3.	Steering

4.	Cornering

5.	Understeer/oversteer

6.	Intersection exercise

C.	Collision Evasion

1.	Weight Transfer

2.	Braking

3.	Turning

4.	Backing

D.	Optional Exercises

1.	Collision avoidance exercise					         II(a,k)

2.	Skid Car Exercise						         II(a,k)

3.	Code 3/Pursuit Driving					         II(a,j)

4.	Intervention Techniques/Risk Assessment			         II(a,i)

(P.I.T. Maneuver)

5.	Spike Strip Deployment and Policy

Testing: Any student scoring below standard on any exercise, as                           II(b)

established by the presenter, will be remediated, tested until

standard is achieved.

EXTENDING EXPANDED COURSE OUTLINE TO 6 OR 8 HOURS:

I.         BASIC DRIVING PRINCIPLES					             	          II(g)

A.          Throttle Control

1.         Full throttle is total depression of accelerator pedal

2.         Maximum acceleration is accelerating as quickly as possible

to full throttle without losing traction

3.         Increased throttle will widen the arc of the driving line in a turn,

and will increase weight transfer

4.         Decrease of throttle will tighten the arc of the driving line in a turn

B.        Speed Judgment

1.       The ability of a driver to estimate a safe speed for any given situation

2.       Considerations

a.	Road conditions

b.	Type of driving maneuvers

c.	Driver limitations

d.	Vehicle limitations

e.	Weather conditions

3.       Closure rate

a.	Being able to judge the proper rate of deceleration necessary to                 negotiate a curve, avoid a hazard, or stop

C.         Brake Application

1.         Types of braking

a.	Normal

b.	Panic

c.	ABS

d.	Threshold

(1)	0-10 scale

(2)	More cooling time

(3)	Allows speed for longer distance before brake application

(4)	Steering always availability

II.         VEHICLE CARE AND MAINTENANCE				        	II(f)

A.	Pre-shift Vehicle Inspection

1.	General appearance

2.	Tires

a.	Pressure

b.	Wear

3.	Wheels

4.	Lights

5.	Trunk

a.	Spare tire

b.	Fire extinguisher

c.	Jack / Lug wrench

d.	Flares

e.	First aid kit

f.	Blanket

6.	Body damage

7.	Interior

a.	Trash / Debris

b.	Clean windows

c.	Adjust seat and mirrors

d.	Check gauges

e.	Brakes

f.	Secure gear

- Seatbelts

8.	Listen for unusual sounds

III.	VEHICLE OPERATION FACTORS					                II(g)

A.	Operational Tactics and Considerations

1.	Radio

a.	Distracted driving

b.	Broadcast while driving in a straight line

c.	Stay calm and speak clearly

2.	Use of MDT/Computer

a.	Stop in a safe place before using any equipment that requires you   to take your eyes off the road

b.	Use radio for communications of immediate nature

c.	Collisions while operating the MDT/Computer will usually be        found to be preventable

B.	Blocking Access to Intersections

1.	Officer Safety

2.	In pursuit, park off to side and get out of way

3.	No guarantee of safety

C.	Blocking Roadway

1.	Position vehicle appropriately

2.	Lighting

3.	Trunk

4.	Stand away from rear of vehicle

5.	Face traffic when walking

IV.	COURSE REVIEW

A.	Written Examination

1.	Lexipol 316, Code 3 Emergency Vehicle Operation		Policy 316

a.	True/False, multiple-choice, fill-in the blanks

2.	Lexipol 314, Vehicle Pursuit Policy				Policy 314

a.	True/False, multiple-choice, fill-in the blanks

3.	California Vehicle Code					II(f)

B.	Officers must score a minimum of 70% to successfully pass the Written Examination.

Testing:  Any student scoring below standard in any exercise, as established by the presenter will be remediated, tested until standard is achieved.